Table 1. Model Variables, Equation (Description), Dimensions, and Exlanations

| Category | # | Variable | Equation / Value | Dimension | Explanation |
| --- | --- | --- | --- | --- | --- |
| Stock | 1 | In-Use Devices | INTEG(adoption rate-collection rate-removal rate, x) | Devices | Accumulates devices currently in clinical use. Inflows: adoption rate. Outflows: collection rate (end-of-use retrieval), removal rate (failure + certificate expiry). Initial value: 10,000 devices. |
| Stock | 2 | Manufactured Devices | INTEG(raw material input rate+recycled material input rate-market release rate, x) | Devices | Accumulates devices in the manufacturing pipeline. Fed by both virgin and recycled material input. Depleted as devices enter the market. |
| Stock | 3 | Marketed Devices | INTEG(market release rate+release to market rate-adoption rate, x) | Devices | Accumulates devices available for adoption, sourced from both new production and reprocessing. Depleted when hospitals adopt them. |
| Stock | 4 | Returned / Collected Devices | INTEG(collection rate-device disassembly rate-reprocessing flow, x) | Devices | Accumulates devices that have been collected post-use. Splits into disassembly (for material recovery) or reprocessing. |
| Stock | 5 | Reprocessed devices | INTEG(reprocessing flow-release to market rate, x) | Devices | Accumulates devices currently undergoing reprocessing. Released back to market after Reprocessing Time. |
| Stock | 6 | Disassembly | INTEG(disassembly rate-landfill rate-incineration rate-recycling rate, x) | Kg | Accumulates material mass from disassembled devices awaiting end-of-life treatment. Outflows: landfill (15%), incineration (20%), recycling. |
| Stock | 7 | Material recycled | INTEG(recycling rate-"recycled material reused in-domain"-recycled material moving to other domains rate, x) | Kg | Accumulates recovered material available for re-entry into manufacturing or export to other domains. |
| Stock | 8 | Landfill | INTEG(landfill rate, x) | Kg | Cumulative material sent to landfill. One-way stock with no outflow. |
| Stock | 9 | Incineration | INTEG(incineration rate, x) | Kg | Cumulative material is incinerated. One-way stock with no outflow. |
| Flow | 10 | adoption rate | Marketed Devices/Release Time | Devices/Month | Devices adopted by hospitals from the marketed stock after a release delay. DELAY N with order 2 represents distributed adoption lags across hospital procurement cycles. |
| Flow | 11 | market release rate | Manufactured Devices/Production Time | Devices/Month | New devices entering the market after production. DELAY3 represents the manufacturing pipeline with a third-order delay. |
| Flow | 12 | collection rate | "In-Use devices"/"In-use Time" | Devices/Month | Rate at which in-use devices are retrieved. Traceability maturity governs how effectively end-of-use devices are identified and recovered. Low traceability = low collection. |
| Flow | 13 | removal rate | (Device Failure Rate+CERTIFICATE EXPIRATION RATE)*"In-Use devices"/Removal Time | Devices/Month | Devices leaving the in-use stock due to failure or regulatory certificate of expiry. Both pathways create replacement demand. |
| Flow | 14 | reprocessing flow | Reprocessable Proportion*Returned Devices Output Rate | Devices/Month | Share of collected devices routed to reprocessing rather than disassembly. Governed by the reprocessable proportion of the device mix. |
| Flow | 15 | device disassembly rate | Returned Devices Output Rate × (1 − Reprocessable Proportion) | Devices/Month | Collected devices entering disassembly after sorting delay. Fifth-order delay represents multi-step logistics. |
| Flow | 16 | release to market rate | Reprocessed devices/Reprocessing Time*INSTITUTIONAL TRUST | Devices/Month | Reprocessed devices re-entering the market after reprocessing and quality-testing delay. |
| Flow | 17 | disassembly rate | device disassembly rate × AVERAGE MATERIAL INTENSITY | Kg/Month | Converts device flow into material mass entering the Disassembly stock. |
| Flow | 18 | recycling rate | Disassembly Output Rate * Recyclable Proportion | Kg/Month | Material recovered for recycling, modulated by device modularity and traceability. Higher modularity and traceability enable more effective material separation. |
| Flow | 19 | incineration rate | Disassembly Output Rate * (1, Recyclable Proportion) * "INCINERATION SHARE OF NON-RECYCLABLE" | Kg/Month | 20% of disassembled material mass incinerated per disassembly cycle. |
| Flow | 20 | landfill rate | Disassembly Output Rate * (1, Recyclable Proportion) * (1, "INCINERATION SHARE OF NON-RECYCLABLE") | Kg/Month | 15% of disassembled material mass sent to landfill per cycle. |
| Flow | 21 | recycled material input rate | "recycled material reused in-domain"/AVERAGE MATERIAL INTENSITY | Devices/Month | Material recovered in-domain converted back to device-equivalent units for re-entry into manufacturing stock. |
| Flow | 22 | raw material input rate | MAX(0, DEMAND, recycled material input rate, release to market rate) | Devices/Month | Virgin material input fills the gap between total demand and what recycled input already covers. MAX(0,…) prevents negative values. |
| Flow | 23 | recycled material reused in-domain | Material recycled/Material Recycling Time*"Reusable In-domain" | Kg/Month | Fraction of recycled material re-entered into the same domain's manufacturing. Governed by in-domain reusability proportion. |
| Flow | 24 | recycled material moving to other domains rate | Reusable in Other Domain*Material recycled/Material Recycling Time | Kg/Month | Recycled material exported to other sectors. Reduces material available for in-domain reuse. |
| Auxiliary | 25 | Reprocessable Proportion | MIN REPROCESSABLE PROPORTION + ((MODULARITY + INSTITUTIONAL TRUST+TRACEABILITY MATURITY)/3) * (MAX REPROCESSABLE PROPORTION - MIN REPROCESSABLE PROPORTION) | Dmnl | Sare of collected devices that can be routed to reprocessing rather than to dissasembly. |
| Auxiliary | 26 | Recyclable Proportion | MIN RECYCLABLE PROPORTION + MODULARITY * (MAX RECYCLABLE PROPORTION - MIN RECYCLABLE PROPORTION) | Dmnl | Share of dissasembled material mass that can be recovered through recycling. |
| Auxiliary | 27 | Reusable In-domain | TRACEABILITY MATURITY*"Base In-domain Reuse Rate" | Dmnl | Share of recycled material that re-enters manufacturing within the medical devices domain. |
| Auxiliary | 28 | Reusable in Other Domain | 1-"Reusable In-domain" | Dmnl | Share of recycled material that leaves the medical devices domain for use in other domain |
| Auxiliary | 29 | Base In-domain Reuse Rate | 0.65*0.6*DECONTAMINATION COMPLIANCE RATE | Dmnl | Maximum share of recycled material that could be reused in-domain under full traceability |
| Auxiliary | 30 | Returned Devices Output Rate | "Returned/ Collected Devices"/Collection Time | Devices/Month | Rate at which collected devices are sorted and leave the returned/ collected stock |
| Auxiliary | 31 | Disassembly Output Rate | Disassembly/Dissasembly Time | Kg/Month | Rate at which dissasembled material mass leaves the dissasembly stock for end-of-life treatment |
| Auxiliary | 32 | Device Failure Rate | BASE FAILURE RATE * (1, TRACEABILITY MATURITY * 0.3)* (1 + (1, REGULATORY STRINGENCY LEVEL) * 0.5) | Dmnl | Fractional rate at which in-use devices fail. Traceability reduces failure by enabling early detection, weak regulation increases it by allowing lower-quality devices to remain in circulation. |
| Auxiliary | 33 | Reprocessing Risk Factor | BASE REPROCESSING RISK *REPROCESSING CYCLES*(1- REGULATORY STRINGENCY LEVEL*0.7)*(1-TRACEABILITY MATURITY*0.3) | Dmnl | Per-device infection risk introduced at reprocessing re-entry. Regulatory stringency is the dominant reducer (weight 0.7), traceability plays a secondary role (0.3). Cycles scale risk proportionally. |
| Auxiliary | 34 | Hospital Infections | (Device Failure Rate*market release rate+ Reprocessing Risk Factor * release to market rate)/adoption rate | Dmnl | Outcome variable. Two additive pathways: (1) failures in the active fleet, (2) infection risk from reprocessed devices re-entering the market. Accumulates both pathways annually. |
| Auxiliary | 35 | Material Price Stability | 1, (1, TRACEABILITY MATURITY) * 0.2 | Dmnl | Proxy for supply chain price predictability. Traceability of material flows reduces price volatility. Range: 0.8 (zero traceability) to 1.0 (full traceability). |
| Auxiliary | 36 | Supply Disruption Risk | MAX(0, 1, REGULATORY STRINGENCY LEVEL * 0.5, INSTITUTIONAL TRUST * 0.5) | Dmnl | Risk of supply chain disruption. |
| Auxiliary | 37 | Cost Resilience | (Cost Saving / Single-Use Cost) × Material Price Stability × (1 − Supply Disruption Risk) | Dmnl | Outcome variable (0-1). Measures how much of the single-use cost baseline is saved through circular pathways, weighted by price stability and supply security. All three factors must hold simultaneously. |
| Auxiliary | 38 | Single-Use Cost | raw material input rate * UNIT COST PER DEVICE NEW + UNIT DISPOSAL COST * (incineration rate + landfill rate) | Euro/Month | Baseline annual cost if all devices were single-use: virgin material procurement + waste disposal costs. |
| Auxiliary | 39 | Circular Cost | recycled material input rate * COST OF RECYCLED MATERIAL * AVERAGE MATERIAL INTENSITY+ release to market rate * UNIT COST REPROCESSING+ ("recycled material reused in-domain"+ recycled material moving to other domains rate)* COST OF RECYCLED MATERIAL | Euro/Month | Annual cost of the circular pathway: reprocessing costs + cost of recycled material flows. |
| Auxiliary | 40 | Cost Saving | "Single-Use Cost"-Circular Cost | Euro/Month | Annual saving from circular versus single-use pathway. MAX(0,…) prevents negative values during model initialisation. |
| Auxiliary | 41 | Waste Generated | incineration rate+landfill rate | Kg/Month | Outcome variable. Total material disposed of annually via incineration and landfill, excluding recycled material. |